Experiment No.: 2

Build a simple network topology and configure it for static routing protocol

- Aim: Build a simple network topology and configure it for static routing protocol using packet tracer.

- Objectives: To introduce concepts and fundamentals of network topologies and their configurations.

- Outcomes: The learner will be able to

- Analyze the functioning of various networking devices.

- Use the simulation tool Cisco Packet Tracer for building networks.

- Recognize the need for topology.

- Hardware/Software required: Cisco Packet Tracer

- Theory

Setting the topology

- Drag and drop the router/switch/computer from the bottom of the screen

2. Select end devices from the bottom left-hand corner and drag it to the sandbox screen.

3.  Select connections from the same bottom left-hand corner. When you connect like-devices (Such as a router and computer)  you use a crossover cable, so you should select Automatically choose connection from the second menu to the immediate right. Click on PC0 to Switch0 and  Router0.

- Repeat step number 3 to connect PC1,PC2,PC3, and Switch0 to Router0.

Configuring the router in packet tracer

- Next we have to open the Ethernet ports to allow communication. Although they are physically connected, they are in a state that is known as being in administrative shut down. Now click on the CLI tab to access the configuration menu.

--- System Configuration Dialog ---

Continue with configuration dialog? [yes/no]: n

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#ip address 192.168.1.1   255.255.255.0

Router(config-if)#exit

Router(config)#interface Serial2/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial2/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial2/0, changed state to up

Router(config-if)#ip address 20.0.0.1 255.0.0.0

Router(config-if)#exit

Router(config)#ip route 192.168.2.0    255.255.255.0    20.0.0.2

Router(config)#end

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

%SYS-5-CONFIG_I: Configured from console by console

Add network

Go to Config >settings save configuration of static routing.

Computer configuration

- Click on PC0 and go to Desktop

- Choose IP configuration and configure IP address as 192.168.1.2 , click on subnet mask, subnet mask address will come automatically.

- Default Gate way is Rourt0 IP address, configure this address as 192.168.1.1.

- Close configuration.

- Repeat step 1 to Step 4 to configure Ip addresses to PC1 to PC3.

Router1 Configuration

--- System Configuration Dialog ---

Continue with configuration dialog? [yes/no]: n

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#ip address 192.168.2.1   255.255.255.0

Router(config-if)#exit

Router(config)#interface Serial2/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial2/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial2/0, changed state to up

Router(config-if)#ip address 20.0.0.2 255.0.0.0

Router(config-if)#exit

Router(config)#ip route 192.168.1.0    255.255.255.0    20.0.0.1

Router(config)#end

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

%SYS-5-CONFIG_I: Configured from console by console

Static Routing

Add network

Go to Config >settings save configuration of static routing.

Computer configuration

- Click on PC4 and go to Desktop

- Choose IP configuration and configure IP address as 192.168.2.2 , click on subnet mask, subnet mask address will come automatically.

- Default Gate Way is Rourt1 IP address, configure this address as 192.168.2.1.

- Close configuration.

Repeat step 1 to Step 4 to configure Ip addresses to PC5 to PC7.

Click on PC0 and then click PC7. On the lower right of the screen you will see a message box that says “Successful.”

- Output Analysis:

(Students should write output analysis based on the working of different topology and different networking devices used in simulation. Specify each scenario explicitly with output analysis)

- Additional Learning:

(Students should write additional learning on their own based on what additionally they learnt after performing the experiment)

- Conclusion: (Students should write conclusion on their own)

- Viva Questions:

- What is the difference between star topology and bus topology?

- State advantages and disadvantages of each of star, bus, ring, mesh.

- Perform the experiment for ring topology and draw your own conclusion.

- State the working difference between hub and switch.

- References:

  - A.S. Tanenbaum, “Computer Networks”, Pearson Education, (4e)

  - B.A. Forouzan, “Data Communications and Networking”, TMH (5e)

- James F. Kurose & K W Ross: Computer Networking: A Top Down Approach, Pearson Education (LPE).

  - https://nptel.ac.in/content/storage2/courses/106105080/pdf/M5L1.pdf

  - https://nptel.ac.in/content/storage2/courses/117105076/pdf/5.1%20Lesson%2015.pdf

  - https://www.netacad.com/courses/packet-tracer

  - https://www.youtube.com/watch?v=A7kOCHdfYtw

  - https://www.youtube.com/watch?v=PmDmRbz7EHM