Experiment No.: 6

Setup a network and configure IP addressing, subnetting, Masking using Cisco Packet Tracer

- Aim: Setup a network and configure IP addressing, Subnetting, Masking using Cisco Packet

Tracer

- Objectives: To introduce concepts of configurations IP addressing, Subnetting and Masking.

- Outcomes: The learner will be able to

- Analyze the functioning of various networking devices.

- Use the simulation tool Cisco Packet Tracer for building networks.

- Recognize the need for IP addressing, Subnetting and Masking

- Hardware/Software required: Cisco Packet Tracer

- Theory

Setting a subnetwork

- Drag and drop the router/switch/computer from the bottom of the screen.

- Select end devices from the bottom left-hand corner and drag it to the sandbox screen.

- Select connections from the same bottom left-hand corner. When you connect like-devices (Such as a router and computer)  you use a crossover cable, so you should select Copper straight-Through, Click on PC0 to Switch0 and  Router0.

- Repeat step number 2to connect PC1,PC2,PC3, and Switch1 to Router0.

To connect router0 and router1, serial connection is required, Click on router0, First turn off router shown by arrow.

Select WIC 1T interface and Place it to slot shown by arrow.

Repeat same procedure for router1

Configuring the router in packet tracer

- Next we have to open the Ethernet ports to allow communication. Although they are physically connected, they are in a state that is known as being in administrative shut down. Now click on the CLI tab to access the configuration menu.

Router0

--- System Configuration Dialog ---

Continue with configuration dialog? [yes/no]: n

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

ip address 192.168.1.1 255.255.255.0

Router(config-if)#

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

ip address 192.168.2.1 255.255.255.0

Router(config-if)#ip address 192.168.2.1 255.255.255.128

Router(config-if)#

Router(config-if)#exit

Router(config)#interface Serial0/3/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to down

Router(config-if)#no ip address

Router(config-if)#ip address 192.168.2.225 255.255.255.128

Router(config-if)#no ip address

Router(config-if)#ip address 192.168.2.225 255.255.255.128

Router(config-if)#ip address 192.168.2.225 255.255.255.252

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

Router con0 is now available

Press RETURN to get started.

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#ip route 192.168.2.128 255.255.255.192 192.168.2.226

Router(config)#ip route 192.168.2.192 255.255.255.224 192.168.2.226

Router(config)#

Router(config)#end

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router con0 is now available

Press RETURN to get started.

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

Router1

--- System Configuration Dialog ---

Continue with configuration dialog? [yes/no]: n

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

no ip address

Router(config-if)#ip address 192.168.2.129 255.255.255.0

Router(config-if)#no ip address

Router(config-if)#ip address 192.168.2.129 255.255.255.0

Router(config-if)#ip address 192.168.2.129 255.255.255.192

Router(config-if)#

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#no shutdown

Router(config-if)#shutdown

Router(config-if)#no shutdown

Router(config-if)#shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to administratively down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to administratively down

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to down no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

ip address 192.168.2.193 255.255.255.192

Router(config-if)#ip address 192.168.2.193 255.255.255.224

Router(config-if)#

Router(config-if)#exit

Router(config)#interface Serial0/3/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

ip address 192.168.2.226 255.255.255.192

% 192.168.2.192 overlaps with FastEthernet0/1

Router(config-if)#ip address 192.168.2.226 255.255.255.252

Router(config-if)#

Router(config-if)#exit

Router(config)#interface Serial0/3/0

Router(config-if)#

Router con0 is now available

Press RETURN to get started.

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#ip route 192.168.1.0 255.255.255.0 192.168.2.225

Router(config)#ip route 192.168.2.0 255.255.255.128 192.168.2.225

Router(config)#

Router(config)#end

Router#copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router con0 is now available

Press RETURN to get started.

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

Static Routing

Go to Config >settings save configuration of static routing for Router0 and Router1.

Configuring the PC in packet tracer

Router Configuration:

Router0

--- System Configuration Dialog ---

Continue with configuration dialog? [yes/no]: n

Press RETURN to get started!

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

ip address 192.168.1.1 255.255.255.0

Router(config-if)#

Router(config-if)#exit

Router(config)#interface FastEthernet0/0

Router(config-if)#

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

ip address 192.168.2.1 255.255.255.0

Router(config-if)#no ip address

Router(config-if)#ip address 192.168.2.1 255.255.255.0

Router(config-if)#ip address 192.168.2.1 255.255.255.128

Router(config-if)#

Router(config-if)#exit

Router(config)#interface Serial0/3/0

Router(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to down

Router(config-if)#ip address 192.168.2.225 255.255.255.128

Router(config-if)#ip address 192.168.2.225 255.255.255.252

Router(config-if)#

Router(config-if)#exit

Router(config)#interface Serial0/3/0

Router(config-if)#

- Output Analysis:

(Students should write output analysis based on the working of different topology and different networking devices used in simulation. Specify each scenario explicitly with output analysis)

- Additional Learning:

(Students should write additional learning on their own based on what additionally they learnt after performing the experiment)

- Conclusion :

(Students should write conclusion on their own)

- Viva Questions:

- State advantages and disadvantages of each of bluetooth

- Name different layers of Bluetooth protocol stack.

- What is piconet and scatternet.

- References:

- A.S. Tanenbaum, “Computer Networks”, Pearson Education, (4e)

- B.A. Forouzan, “Data Communications and Networking”, TMH (5e).

- James F. Kurose & K W Ross: Computer Networking: A Top Down Approach, Pearson Education (LPE).